## CD 20 Technical  Information

Written by Dany van-Gelder

Date 26/July/2004

CD20 is build of two electronic boards

Digital board – it is the same digital board that used in all CD series 5 line or the digital board which is used in the synqnet drive, this document does not cover the digital board

Power board – design for 20 Amp rms and 40 Amp rms for two seconds, this document cover all the technical info for the ordering part number and power board information.

Ordering Information

Example:

CE20561 – CD series 5, 20 Amp rms Cont, Incremental encoder feedback, SERCOS interface

CR20560 – CD series 5, 20 amp rms Cont, resolver feedback, Analog Interface

LB20565 – Synqnet Drive, 20 amp rms Cont, sine encoder feedback, Synqnet Interface

## Input Power Filtering

- The SERVOSTAR CD electronic system components require EMI filtering in the input power leads to meet the conducted emission requirements for the industrial environment. This filtering blocks conducted-type emissions from exiting onto the power lines and provides a barrier for EMI on the power lines.

- Care must be taken to adequately size the system. The type of filter is based on the voltage and current rating of the system the incoming line is three-phase. These filters are mounted as close to the incoming power as possible so noise is not capacitively coupled into other signal leads and cables. These lines may be noisy and should be separated from other sensitive cabling to avoid unwanted coupling of noise.  Several manufacturers of these filters are listed below. They should be able to recommend the best filter design for most typical motor control applications. Danaher Motion has also provided specific filters recommendations that adequately attenuate the conducted noise to levels well below the CE limits. The implementation of the EMI filter should be done in accordance with the following guidelines:

— Filter should be mounted on the same panel as the drive.

— Filter should be mounted as close as possible to incoming cabinet power.

— When mounting the filter to the panel, remove any paint or material covering. Use an unpainted metallic back panel, if possible.

— Filters are provided with an earth connection. All ground connections are tied to ground.

— Filters can produce high leakage currents. Filters must be earthed before connecting the supply!

- — Filters should not be touched for a period of 10 seconds after removing the supply.

|  | Recommended EMI Line Filter for CD20 | Danaher Motion Part # |
| --- | --- | --- |
|  | Schaffner FN3258-30-47 http://www.schaffner.com Schaffner Holding AG Nordstrasse 11 CH-4542 Luterbach Switzerland T +41 32 6816 626 F +41 32 6816 641 |  |
|  | CORCOM http://www.corcom.com Telephone 847-680-7400 For Customer Service please call 1-800-468-2023 FAX 847-680-8169 Postal address 844 E. Rockland Road Libertyville, Illinois 60048 |  |
|  | Fair-Rite http://www.fair-rite.com Telephone (888) FAIRRITE (324-7748) or (845)895-2055 FAX (888) FERRITE (337-7483) or (845) 895-2629 Postal address PO Box J, 1 Commercial Row Wallkill, NY 12589 |  |
|  | Micrometals |  |
|  | Magnetics |  |
|  | Phoenix |  |
|  | FerriShield |  |
|  | Filter Concepts |  |

# Wiring

## Outline Dimensions

## Cx20  System Wiring Diagram (CD sereis 5 line)

## Lx20  System Wiring Diagram (synqnet line)

## Hardware Specifications

| Amplifier Model |  | Cx20 |  |  |
| --- | --- | --- | --- | --- |
| Unit Weight | Lbs./Kgs. | 8.8 / 4 |  |  |
| Mounting Hardware | English (Metric) | 10-32 (M4) |  |  |
|  | Applied Torque | 20 lb-in. (2.26 Nm.) |  |  |
| Connection Hardware | Line Screw Size/Torque | M3.5/12 lb-in. (1.35 N-m) |  |  |
|  | BUS Screw Size/Torque |  |  |  |
|  | Motor Screw Size/Torque |  |  |  |
|  | Ground Screw Size/Torque |  |  |  |
| Wire Size (AWG#) | Control Logic (AWG/ mm2) | 16 / 1.31 |  |  |
|  | Configurable I/O wire gauge | 22-18 AWG (0.3-0.75 mm2) Ferrules recommended: 18 AWG Type H0 - 75/14 Weidmuller 4629.0 or equivalent 20 AWG Type H0 - 5/14 Weidmuller 6907.0 or equivalent 22 AWG Type H0 – 34/12 Weidmuller 90257.7 or equivalent |  |  |
|  | Regen (AWG/ mm2) C B+ | 10 / 5.26 |  |  |
|  | Motor Line (AWG/ mm2) Main Input Ma Mb Mc L1 L2 L3 | 10 / 5.26 |  |  |
| Clearance Distance | Side-to-Side | 0.5 in (12.7 mm) |  |  |
|  | Top/Bottom | 2.5 in (63.5 mm) |  |  |
| Mating Connector Hardware | CK100 Kit | Includes: C1, C2, C4, C7 (plus 2 ft./0.69 m. of stranded bus ribbon), C8 |  |  |
|  | C1 | D9 Female EDA 8S0009SS402S25 Connector 9 pins D type for Cable solder |  |  |
|  | C2 | D25 Male EDA 8S0025SS402525 Connector 25 pins D Type For Cable solder |  |  |
|  | C3 | Danaher Motion #: A-93899-013 Vendor Info: Weidmuller BL3.5/13 Cat.No. 161574 |  |  |
|  | Connector Screw Torque | 2.25 lb-in. (0.25 N-m) |  |  |
|  | C4 | D9 male or Sercos or RJ45 for synqnet EDA 8S0009SS402S25 Connector 9 pins D type for Cable solder Sercos matting connector come w/ the sercos cable RJ45 shielded Ethernet cable |  |  |
|  | C7 | 10 Pin (0.1”x0.1”) Female ribbon |  |  |
|  | C8 | D9 male EDA 8S0009SS402S25 Connector 9 pins D type for Cable solder |  |  |
|  | C9 | 20 PIN connector HIROSE FX2BA-20SA-1.27R |  |  |
|  | Regen (AWG/ mm2) C B+ | Connector: 2 x PIN Molex MiniFit Molex P/N 42816-0212 *Terminal (Pins): Molex P/N 42815-0031 |  |  |
|  | Motor Line (AWG/ mm2) Main Input Ma Mb Mc L1 L2 L3 | Connector: 6 x PIN Molex MiniFit Molex P/N 42816-0612 *Terminal (Pins): Molex P/N 42815-0031 |  |  |
|  | 24 V Logic (optional) | Danaher Motion #:A-81014-002 Vendor Info: PCD ELFP02210 |  |  |

* Molex Crimp tooling P/N 63811-1500 for terminal pins

## Electrical Specifications

| - Product Model |  | - Cx20 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Main Input Power | - Voltage (VACL-L) Nominal 10% | - 180 to 230 3 only |  |  |  |
|  | - Line Frequency Hz | - 47-63 |  |  |  |
|  | - Input power at 180 VAC L-L | - 5.3 kW |  |  |  |
|  | - Input power at 208 VAC L-L | - 6.2 kW |  |  |  |
|  | - Input power at 230 VAC L-L | - 6.8 kW |  |  |  |
|  | - Continuous Current (A) | - 20amps/phase |  |  |  |
|  | - Peak Current (A) for 2s | - 40amps/phase |  |  |  |
|  | - Line Fuses (FRN-R, LPN, or equiv.) | - 25 |  |  |  |
| - Logic Input Power | - +24 VDC Ext. Logic Voltage (V) | - 22 to 27 |  |  |  |
|  | - +24 VDC Ext. Logic Current (amps sink) | - 1.5 |  |  |  |
|  | - +24 VCD Ext. Logic Current (amps max surge) | - 2.6 |  |  |  |
| - SoftStart | - Max. Surge Current (A) | - 200 |  |  |  |
|  | - Max. Charge Time (msec) | - 50 |  |  |  |
| - Protection Functions | - Fault Contact Rating (A) | - 1 |  |  |  |
|  | - Fault Contact Closing Period (ms) | - Close = 3 ms, Open = 2 ms |  |  |  |
|  | - Over-Temperature Trip (C/F) | - 80/176 |  |  |  |
| - Rated Main Output - (Ma, Mb, Mc) | - Continuous Power at 180 VAC L-L Line Input (45C (113F) Ambient) | - 4 kW |  |  |  |
|  | - Continuous Power at 230 VAC L-L Line Input (45C (113F) Ambient) | - 5.2 kW |  |  |  |
|  | - Continuous Current (ARMS) | - 20 |  |  |  |
|  | - Peak Current (Arms) for 2 sec | - 40 |  |  |  |
|  | - PWM Frequency (kHz) | - 8 |  |  |  |
|  | - PWM Motor Current Ripple (kHz) | - 16 |  |  |  |
|  | - Form Factor (rms/avg) | - 1.01 |  |  |  |
| - Protective Functions | - Under-Voltage Trip (nominal) (VDC) | - 100 |  |  |  |
|  | - Over-Voltage Trip (VDC) | - 400 |  |  |  |
|  | - Over-Temperature Trip (C/F) | - 80/176 |  |  |  |
|  | - Internal heat dissipation (watts) | - 330 |  |  |  |
| - Environment | - Operation temperature (C/F) | - 5/41 to 45/113 |  |  |  |
|  | - Storage temperature (C/F) | - 0/32 to 70/158 |  |  |  |
|  | - Ambient humidity (%) | - 10 to 90 |  |  |  |
|  | - Atmosphere | - without corrosive gasses or dust |  |  |  |
|  | - Altitude | - Derate 5% per 1000 ft.(300 m) above 3300 ft. (1000 m) |  |  |  |
|  | - Vibration (g) | - 0.5 |  |  |  |

## Regen Information

| - Product Model |  | - Cx20 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - External Shunt Regulator | - Peak current (amps) |  | - 45 |  |  |  |  |
|  | - Minimum resistance (ohms) |  | - 4 @ 190VBUS 8 @ 320VBUs |  |  |  |  |
|  | - watts |  | - 1kW |  |  |  |  |
|  | - BUS Voltage (nominal) (VDC) |  | - 325 |  |  |  |  |
|  | - VHYS (Regen circuit turn-off) (VDC) |  | - 375 |  |  |  |  |
|  | - VMAX (Regen circuit turn-on) (VDC) |  | - 385 |  |  |  |  |
| - External Regen Kits | - TBD |  | -  | -  |  | -  |  |