G & L Motion Control Inc

PiC Application Note

Document Number:  AN000013

Topic:  Software/EPROM Compatibility Charts

Software/EPROM Compatibility Charts	3

PiCPro Executive Firmware Release Dates	5

CPU module identification chart using module LEDs	7

Software/EPROM Compatibility Chart for PiC900 186 CPUs, 6 MHz, 8 MHz and 16 MHz	9

Software/EPROM Features for PiC900 186 CPUs, 6 MHz, 8 MHz and 16 MHz	10

Software/EPROM compatibility CHART for PiC9, PiC90 186 cpus, 8 MHz and 16 MHz	11

Software/EPROM Features for PiC9, PiC90 186 cpus, 8 MHz and 16 MHz	12

Software/EPROM COMPATIBITY CHART for 486 DX and 486 SX cpus	13

Software/EPROM Features for 486 DX and 486 SX cpus	14

Software/EPROM Compatibility Chart for 911 and 912 (186) cpus	15

Software/EPROM Features for 911 and 912 (186) cpus	16

Software/EPROM Compatibility Chart for 9011 and 9012 (186) cpus	17

Software/EPROM Features for 9011 and 9012 (186) cpus	18

Software/FLASH Compatibility Chart for 941, 943, 945 and 947 (486) CPUs	20

Software/FLASH features for 941, 943, 945 and 947 (486) CPUs	22

Software/FLASH Compatibility Chart for 9041 and 9043 (486) cpus	24

Software/FLASH Features for 9041 and 9043 (486) cpus	25

Software/FLASH Compatibility Chart for MMC and MMC Plus (486) cpus	27

Software/FLASH Features for MMC (486) cpus	28

Software/FLASH Compatibility Chart for MMC for PC SERCOS (486) cpu	29

Software/FLASH Features for MMC for PC SERCOS (486) cpu	30

Software/FLASH Compatibility Chart for MMC for PC Analog (486) cpu	31

Software/FLASH Features for MMC for PC Analog (486) cpu	32

MMC/PiC90/900 hardware COMPATIBILITY ISSUES	34

CPU modules	34

MMC CPU modules	34

PIC90/900 CPU modules	35

CSM Modules on PiC 900 Racks	36

PiC REMOTE RACK COMMUNICATIONS	36

PiC Communication Modules	36

PiC Feedback Modules	37

PiC Analog Output Modules	37

NOTE

Progress is an on going commitment at G & L Motion Control Inc. We continually strive to offer the most

advanced products in the industry; therefore, information in this document is subject to change without

notice. The illustrations and specifications are not binding in detail. G & L Motion Control  Inc shall not be

liable for any technical or editorial omissions occurring in this document, nor for any consequential or

incidental damages resulting from the use of this document.

DO NOT ATTEMPT to use any G & L Motion Control Inc product until the use of such product is completely

understood. It is the responsibility of the user to make certain proper operation practices are

understood. G & L Motion Control Inc  products should be used only by qualified personnel and for the

express purpose for which said products were designed.

Should information not covered in this document be required, contact the Customer Service

Department, G & L Motion Control Inc, 672 South Military Road, P.O. Box 1960, Fond du Lac, WI 54936-1960.

G & L Motion Control Inc can be reached by telephone at (920) 921–7100.

Software/EPROM Compatibility Charts

This application note contains charts on EPROM/PiCPro compatibility for all versions of PiCPro beginning with 3.5.  There are two sets of charts for each processor type: One lists features that were included in each version of PiCPro, and one lists whether or not particular versions of PiCPro will work with a particular EPROM.  The processor types are grouped into PiC900 186, 486, 911/912, 941/943/945/947, PiC9/90 186, 9011/9012, 9041/9043 and MMC.

The 941/943/945/947/9041/9043/MMC processors do not have EPROMs.  Instead, the system and diagnostic information is stored in flash memory resident on the processor.  The flash memory can be loaded using the PiCPro for DOS software utility called SENDHEX, which is included on the PiCPro Utilities disk; this utility is part of PiCPro for Windows version 11.0 and above.

Here are some additional notes that are of importance:

- When upgrading PiCPro for DOS, you must start PiCPro the first time with the /u command line switch to rebuild all UDFBs designated by SET PICLIB= statement in batch file.  For PiCPro for Windows versions 10.0 and up, a recompile of all UDFBs found is sufficient.

- Restored files created by a Module Binary Dump assume you have the proper EPROMs for the version of PiCPro you are running.

- You can not share the standard libraries from one version (e.g. motion.lib, arith.lib, timers.lib) with another version of PiCPro.  If you try to, PiCPro will not properly read the libraries.

- Some motion.lib/EPROM combinations require a power cycle of the control before they will work.  For example, going from 4.0 to 6.0.

- If PiCPro for DOS is used to make a binary file, and the /f (force soft bit memory) switch is used to start PiCPro, the binary file that results will only work in a processor with EPROMs of an equivalent version or higher.  PiCPro for Windows has this option on its compile settings page.

- If you want to use any of the features listed in a version, you must use at least that level of EPROM/PiCPro or higher.

- It is possible that some combinations that are Yes on the chart may not work in some applications as this was not an exhaustive test of all features.  Conversely, it is possible that some combinations that are not Yes on the chart may work, for example if the ladder is only simple I/O.

- PiCPro LDOs are not backward compatible with the exception of PiCPro 9.2 or 9.3 and PiCPro for Windows 10.0.  The 10.0 ladder is the same format as the 9.2/9.3 ladder so that a customer can run the same source on both versions.  The same source can also be used with PPfW 11.0 and PPfW 12.0.

- PiC9 and PiC90 processors must be used with PiCPro version 5.1 through 10.2.

- 911 and 912 processors must be used with PiCPro version 7.0 through 10.2.

- 943, 945 CPU’s w/o Block I/O must be used with PiCPro version 8.0 or higher.

- 943, 945 CPU’s w/ Block I/O must be used with PiCPro version 9.0 or higher

- 9011 and 9012 processors must be used with PiCPro version 9.0 through 10.2.

- 9041, 9043, 941and 947 processors must be used with PiCPro version 9.2 or higher.

- MMC processors must be used with PiCPro version 10.2 or higher.

- PiCPro for Windows should not be used to open LDOs created in PiCPro Version 8.2x or lower.  A UDFB update (/U) is required in 9.3 before moving forward to PPfW.  Standard libraries in 9.3 are the same as in PPfW 10.0 making it possible to go back and forth from PPfW 10.X to PPfD 9.X without having to rebuild UDFBs.

- 186 CPU firmware cannot be upgraded beyond Dec 98 releases.  PiCPro will still be compatible; however, future enhancements and features will not be supported.

- PiCPro for Windows version 11.0 is only for CPUs with 486DX processors (as 11.0 introduced the 32-bit motion library) so it will not support the older CPUs (i.e., none of the 186 or 486SX processors). Subsequent PiCPro for Windows versions will also be targeted for the 486DX-class processors (with an integrated math coprocessor). The 486DX 503-03814-XX Turbo2 processor is supported by PiCPro for Windows version 11.0 but it is one of the older style modules (the 94x series is the current PiC900 family).

- MMC Plus CPU’s must be used with PiCPro V14.0 or higher.

- MMC-S8 Plus (8-axis) SERCOS must be used with V14.0-SP2 or higher.

- MMCD must be used with V15.0 or higher.

The following table of PiCPro release dates can be used to zero in on what version may be in various systems.  Note that EPROM revision numbers can be identified by performing a Processor stAtus from the PiCPro module menu or Oline | Status from the PPfW menu.  Also, note that the Compiler version can be identified by opening the .LDO or .SRV in a text editor such as MS Notepad.

| PiCPro Executive Firmware Release Dates |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PiCPro Version | .LDO Format Version | .SRV Format Version | .SRC Format Version | Release # Date | EPROM Revision from Processor Status | Features Added |
| 3.5 |  |  |  | 15 Aug 91 | 100 | 256K Application Program Memory 32 Point Output Cards 19.2K Baud User Port |
| 4.0 |  |  |  | 06 Jan 92 | 100 | Math Co-Processor Serial Communications Module User Defined Function Blocks (UDFBs) |
| 4.1 |  |  |  | 15 Mar 92 | 100 | Minor Update |
| 5.0 |  |  |  | 20 Dec 92 | 100 | Auto Baud Rate of PiCPro Port RAMDISK Functions |
| 5.1 |  |  |  | 26 Mar 93 | 100 | Minor Update |
| 5.1.1 |  |  |  | 01 Apr 93 | 100 | Minor Update |
| 6.0 | 5 | 4 |  | 14 Nov 93 | 200 | Extended Application Memory BAT_Yes Function On-Line Edit |
| 6.1 | 5 | 4 |  | 24 Nov 93 | 200 | Minor Update |
| 6.2 | 5 | 4 |  | 13 Mar 94 | 200 | Minor Update |
| 6.3 | 5 | 4 |  | 16 Oct 94 | 200 | Minor Update |
| 7.0 | 5 | 4 |  | 10 Apr 95 | 300 | Supports 911, 912 PiCPro over ARCNET Flash Disk Facilities Extended Data Memory Tasks |
| 7.1 | 5 | 4 |  | 09 Jul 95 | 300 | Minor Update |
| 8.0 | 6 | 4 | 4 | 21 Jan 96 | 400 | Supports 943, 945 On-Line Edit of UDFBs Animation, View List and Forcing Enhancements |
| 8.1 | 6 | 4 | 4 | 05 May 96 | 400 | Minor Update |
| 9.0 | 6 (7 if Block I/O) | 5 | 4 | 14 Jul 97 | 500 | Supports 9011, 9012 Project Manager 32 axes support Load Flash memory without stopping scan Block I/O introduction (4-20 amp & analog in) |
| 9.1 | 6 (7 if Block I/O) | 5 | 4 | 24 Nov 97 | 500 | Supports 943, 945, 9011, 9012 w/Block I/O Project Manager enhancements Additional Block I/O (DC I/O (8/8), +/- 10 analog out, 6 channel resolver, stepper encoder module) |
| 9.2 | 6 (7 if Block I/O) | 5 | 4 | 28 Apr 98 | 500 | Supports 9041, 9043, 941, 947 Stepper Axis Module |
| 9.3 | 6 (7 if Block I/O) | 5 | 4 | 16 Nov 98 | 500 | Full compatibility w/PiCPro for Windows Initial SERCOS support Scan Loss fix (problem in all prior versions) TTL interrupt driven task |
| PicPro for Windows 10.0 | 6 (7 if Block I/O) | 5 | 5 | 16 Nov 98 | 500 | Full compatibility w/PiCPro 9.3 for DOS SERCOS support |
| PPfW 10.1 | 6 (7 if Block I/O) | 5 | 5 | 01 Apr 99 | 500 | Windows 98 support SERCOS enhancements FieldBus (DeviceNet) initial (beta) support Ethernet TCP/IP initial (beta) support |
| PiCPro | .LDO | .SRV | .SRC | Release | EPROM | Features Added |
| PPfW 10.2 | 6 (7 if Block I/O) | 6 | 5 | 22 Jun 99 | 500 | MMC Analog servo only (MMC .srv not compatible with previous versions of PiCPro; PPfW has ‘save as’ pre-10.2 for PiC .srv) |
| PPfW 11.0 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) | 6 (7 if MMC SERCOS) | 6 (5 via Save As) | 16 Dec 99 17 Dec 99 | 500 | MMC SERCOS 32-bit motion library Expanded data memory (up to 192K) Ladder configurable I/O (its .ldo not compatible pre-11.0) SERCOS registration New Project Manager Additional Block I/O (AC input 8pt, AC output 8pt, Relay output 8pt) MMC Ethernet and DeviceNet modules PiCPro over Ethernet |
| PPfW 11.0 Service Pack 1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) | 6 (7 if MMC SERCOS) | 6 (5 via Save As) | 6 Mar 00 | 500 | Added Cancel FAST_QUE “holding”. Added WRITE_SV #58 “SERCOS Modulo Value” |
| PPfW 12.0 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) | 8 (6 via Save As) (7 via Save As) | 7 (5 via Save As) (6 via Save As) | 16 Jan 01 | 500 | MMC for PC support Windows 2000 support Hdwr. Declarations Block I/O insert/delete support |
| PPfW 12.0 SP1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) | 8 (9 if SERCOS) (6 via Save As) (7 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 26 Apr 01 | 500 | MMC for PC SERCOS 32 axis support Windows ME support Added SERCOS “Position Error Cyclic Update Offset” |
| PPfW 12.0 SP2 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) | 8 (9 if SERCOS) (6 via Save As) (7 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 26 Oct 01 | 500 | Added 4I/4O Analog Block I/O |
| PPfW 13.0 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 20 May 02 | 500 | Structured Text S-Curve Resumable E-Stop |
| PPfW 13.0 SP1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 5 Aug 02 | 500 |  |
| PPfW 13.0 SP2 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 11 Sep 02 | 500 |  |
| PPfW 13.0 SP3 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 22 Oct 02 | 500 | DLS PID Enhancements J-K Thermocouple |
| PPfW 13.0 SP4 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 19 Mar 03 | 500 |  |
| PPfW 13.0 SPEC5 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 25 Apr 03 | 500 |  |
| PiCPro | .LDO | .SRV | .SRC | Release | EPROM | Features Added |
| PPfW 14.0 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 7 Oct 03 | 500 | MMC Smart Drive MMC Plus Virtual Axis |
| PPfW V14.0 SP1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 12 Dec 03 | 500 | MMC Smart Drive -230 |
| PPfW V14.0 SP2 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 27 Jan 04 | 500 | MMC Smart Drive -230 |
| PPfW V14.1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 18 May 04 | 500 | Password Protect LDO File Upload All SERCOS Slaves Customer Motor Table Support Superimposed Move |
| PPfW V15.0 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 13 Jul 05 | 501 | MMC DIGITAL SMART DRIVE |
| PPfW V15.1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 22 Sep 05 | 502 | MMC DIGITAL SMART DRIVE enhancements |
| PPfW V15.1 SP1 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) | 13 Apr 06 | 502 | Support for 10,000 line encoders. Notch Filter support was added. Added support for new digital drive models. PLS and CAM_OUT functions support MMCD PowerX Support |
| PPfW V15.1 SP2 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) |  | 502 | Change was made to Digital Drive Firmware Only |
| PPfW V15.1 SP3 | 6 (7 if Block I/O) (8 if Conf. I/O or exp. data) (9 if ext. pool) (10 if pass. protect) | 10 (11 if DLS) (12 if Virtual Axis) (6 via Save As) (7 via Save As) (8 via Save As) (9 via Save As) | 7 (8 MMC for PC > 16 slave) (5 via Save As) (6 via Save As) |  | 502 | Change was made to Digital Drive Firmware Only |

CPU module identification chart using module LEDs

PiC90

4 leds		AC power	Std - 3846		Turbo - 3853

DC power	Std - 4035		Turbo - 4036

11 leds		AC power	Std-9011 – 4071	Turbo-9012 - 4073

DC power 	Std-9011 – 4080	Turbo-9012 - 4081

13 leds		AC power 	Std-9041 – 4104	Turbo-9043 - 4125

DC power 	Std-9041 – 4110	Turbo-9043 - 4126

PiC900

1 led		SAVE - 3680	Std - 3510	Turbo - 3638

Turbo2 - 3794 (w/486SX)	Turbo2 - 3814 (w/486DX)

8 leds		Std-911 - 3941			Turbo-912 - 3963

Turbo2-941 - 4111		Turbo2-943 - 4011

Turbo3-945 - 3994		Turbo3-947 – 4112

Legend for compatibility charts

For the following compatibility charts, the leftmost column lists the EPROM Version number, the date it was released, and in parentheses the PiCPro version it was originally released with.  Note apply as follows:

Note 1 = Probably will not work, may cause intermittent communication time-out errors when loading, loss of animation, intermittent loss of scan, and may corrupt data on RAMDISK.

Note 2 = Works OK if you change the hardware declarations for CPU type.  If you do not, and your .LDO is bigger than 64K or uses a numeric processor you will get an error when downloading, and downloading will not finish.

Note 4 = Motion.libs before version 7.0 of PiCPro will not work with 7.0 or higher EPROMs on PiC90 and PiC900 186 processors that use the NPX.  If you are not using the NPX, it works OK.  Versions of PiCPro before 7.0 will not work with 7.0 or higher EPROMs in systems that continually open and close files or serial ports, such as PiCNC.

Note 5 = Not tested.

Note 6 = Only 486DX processors are supported

Note 7 = This feature will work, but may have some limitations

Yes = Yes, this feature is supported.

No = No, this feature is not supported.

PPfD = PiCPro for DOS

PPfW = PiCPro for Windows

| Software/EPROM Compatibility Chart for PiC900 186 CPUs, 6 MHz, 8 MHz and 16 MHz Processor P/N: 502-03680-XX (SAVE), 502-03510-XX (STD), 502-03638-XX (Turbo) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # EPROM: 90853-XX |  | PiCPro 3.5 | PiCPro 4.X | PiCPro 5.X | PiCPro 6.X | PiCPro 7.X | PiCPro 8.X | PiCPro 9.X | PPfW 10.X | PPfW 11.0 and Higher |
| -06 (3.5) 8-19-91 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -07 9-17-91 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -08 12-13-91 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -09 (4.1) 3-25-92 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -10 4-28-92 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -11 (5.0) 12-22-92 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -12 (6.1) 11-16-93 | Download Restore | Note 1 Note 1 | Note 1 Note 1 | Yes Note 1 | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -13 (7.0) 4-12-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -14 8-28-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -15 (8.0) 1-24-96 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -16 (9.0) 7-23-97 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Note 5 Note 5 | Yes Yes | Yes Yes | No Yes |

| Software/EPROM Features for PiC900 186 CPUs, 6 MHz, 8 MHz and 16 MHz Processor P/N: 502-03680-XX (SAVE), 502-03510-XX (STD), 502-03638-XX (Turbo) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # EPROM: 90853-XX | -06 | -07 | -08 | -09 | -10 | -11 | -12 | -13 | -14 | -15 | -16 |
| PiCPro 3.5 256K Application RAM 32 Point Output Cards 19.2K Baud User Port | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes |
| PiCPro 4.X Math Co-Processor Serial Comm. Module UDFBs | No No No | No No No | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes |
| PiCPro 5.X Auto Baud Rate RAM Disk Functions | No No | No No | No No | No No | No No | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 6.X Extended App. Memory BAT_Yes Function On-Line Edit | No No No | No No No | No No No | No No No | No No No | No No No | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes |
| PiCPro 7.X ARCNET Support Flash Disk Facilities Extended Data Memory Tasks | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes |
| PiCPro 8.X On-line Edit UDFB Animation, View Force Enhancement | No No No | No No No | No No No | No No No | No No No | No No No | No No No | No No No | No No No | Yes Yes Yes | Yes Yes Yes |
| PiCPro 9.X Project Manager SERCOS Block I/O | No No No | No No No | No No No | No No No | No No No | No No No | No No No | No No No | No No No | No No No | Yes Yes Yes |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No | No | No | No |
| PPfW 11.0 and higher does not support the 186 processors |  |  |  |  |  |  |  |  |  |  |  |

* PiCPro 10.0 for Windows does not support PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/EPROM compatibility CHART for PiC9, PiC90 186 cpus, 8 MHz and 16 MHz Processor P/N: 502-03846-XX (STD AC), 502-03853-XX (Turbo AC), 502-04053-XX (STD DC), 502-04036-XX (Turbo DC), All PiC9s |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # EPROM: 92967-XX |  | PiCPro 5.X | PiCPro 6.X | PiCPro 7.X | PiCPro 8.X | PiCPro 9.X | PPfW 10.X | PPfW 11.0 and Higher |
| -00 (5.1) 3-19-93 | Download Restore | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -01 4-11-93 | Download Restore | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -02 (6.1) 11-16-93 | Download Restore | Yes Note 1 | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | No Yes |
| -03 (7.0) 4-12-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -04 6-21-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -05 8-28-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -06 (8.0) 1-24-96 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -07 9-27-96 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -08 (9.0) 7-23-97 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -09 (9.1) 11-25-97 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Note 1 Note 1 | Yes Yes | Yes Yes | No Yes |

| Software/EPROM Features for PiC9, PiC90 186 cpus, 8 MHz and 16 MHz Processor P/N: 502-03846-XX (STD AC), 502-03853-XX (Turbo AC), 502-04053-XX (STD DC), 502-04036-XX (Turbo DC), All PiC9s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPROM: 92967-XX | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 | -09 |
| PiCPro 5.X Auto Baud Rate RAM Disk Functions | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 6.X Extended App. Memory BAT_Yes Function On-Line Edit | No No No | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes |
| PiCPro 7.X ARCNET Support Flash Disk Facilities Extended Data Memory Tasks | No No No No | No No No No | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes |
| PiCPro 8.X On-Line Edit UDFB Animation, View, Force Enhancement | No No | No No | No No | No No | No No | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 9.X Project Manager SERCOS Block I/O Flash load w/out stopping scan | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | Yes Yes No Yes | Yes Yes Yes Yes |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No | No |
| PPfW 11.0 and higher does not support the 186 processors |  |  |  |  |  |  |  |  |  |

* PiCPro 10.0 for Windows does not support PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/EPROM COMPATIBITY CHART for 486 DX and 486 SX cpus Processor P/N: 502-03794-XX (Turbo2 SX), 502-03814-XX (Turbo2 DX) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # EPROM: 92668-XX |  | PiCPro 5.X | PiCPro 6.X | PiCPro 7.X | ### PiCPro 8.X | PiCPro 9.X | PPfW 10.X | PPfW 11.0 | PPfW 12.0 and higher |
| -00 1-05-93 | Download Restore | Yes Yes | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -01 (6.0) 11-16-93 | Download Restore | Yes Note 1 | Yes Yes | Note 2 Yes | Note 2 Yes | Note 2 Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -02 4-12-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -03 (7.0) 4-25-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -04 8-28-95 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -05 (8.0) 1-23-96 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -06 (9.0) 7-23-97 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | No Yes |
| -07 (10.1) 4-02-99 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Yes | No Yes |
| -08 (10.2) 4-12-99 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Yes | No Yes |
| -09 (11.0)SP1 2-25-00 | Download Restore | Note 4 Note 4 | Note 4 Note 4 | Note 1 Note 1 | Note 4 Note 4 | Note 5 Note 5 | Note 5 Note 5 | Note 6 Yes | No Yes |

*Note: PiCPro for Windows Version 11.0 is for the 486DX processors (as it uses a 32-bit motion library) so the 3794 (486SX) is not supported but the 3814 (486DX) is supported.

| Software/EPROM Features for 486 DX and 486 SX cpus Processor P/N: 502-03794-XX (Turbo2 SX), 502-03814-XX (Turbo2 DX) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPROM: 92668-XX | -00 | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 | -09 |
| PiCPro 5.X Auto Baud Rate RAM Disk Functions | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 6.X Extended App. Memory BAT_Yes Function On-Line Edit | No No No | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes | Yes Yes Yes |
| PiCPro 7.X ARCNET Support Flash Disk Facilities Extended Data Memory Tasks | No No No No | No No No No | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes |
| PiCPro 8.X On-Line Edit UDFB Animation, View and Force Enhancement | No No | No No | No No | No No | No No | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 9.X Project Manager SERCOS Block I/O Flash load w/out stopping scan | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | Yes Yes No Yes | Yes Yes No Yes | Yes Yes No Yes | Yes Yes No Yes |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No | No | No |
| PPfW 11.0 New Project Manager | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes |
| # PPfW 12.0 and higher does not support these processors |  |  |  |  |  |  |  |  |  |  |

* PiCPro 10.0 for Windows does not support the PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/EPROM Compatibility Chart for 911 and 912 (186) cpus Processor P/N: 502-03941-XX (STD 911), 502-03963-XX (Turbo 912) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| EPROM: 94718-XX |  | PiCPro 7.X | PiCPro 8.X | PiCPro 9.X | PPfW 10.X | PPfW 11.0 and Higher |
| -00 (7.0) 4-10-95 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -01 7-17-95 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -02 8-01-95 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -03 8-28-95 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -04 (8.0) 1-24-96 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -05 9-27-96 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -06 (9.0) 7-23-97 | Download Restore | Note 1 Note 1 | Yes Yes | Yes Yes | Note 5 Yes | No Yes |
| -07 11-25-97 | Download Restore | Note 1 Note 1 | Yes Yes | Yes Yes | Note 5 Yes | No Yes |

| Software/EPROM Features for 911 and 912 (186) cpus Processor P/N: 502-03941-XX (STD 911), 502-03963-XX (Turbo 912) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPROM: 94718-XX | -00 | -01 | -02 | -03 | -04 | -05 | -06 | -07 |
| PiCPro 7.X ARCNET Support Flash Disk Facilities Extended Data Memory Tasks | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes |
| PiCPro 8.X On-Line Edit UDFB Animation, View and Force Enhancement | No No | No No | No No | No No | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| PiCPro 9.X Project Manager SERCOS Block I/O Flash load w/out stopping scan | No No No No | No No No No | No No No No | No No No No | No No No No | No No No No | Yes Yes No Yes | Yes Yes No Yes |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No |
| PPfW 11.0 and higher does not support the 186 processors |  |  |  |  |  |  |  |  |

* PiCPro 10.0 for Windows does not support the PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/EPROM Compatibility Chart for 9011 and 9012 (186) cpus Processor P/N: 502-04071-XX (STD 9011 AC) , 502-04073-XX (Turbo 9012 AC), 502-04080-XX (STD 9011 DC), 502-04081-XX (Turbo 9012 DC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| EPROM: 96742-XX |  | PiCPro 9.X | PPfW 10.X | PPfW 11.0 and Higher |
| -00 (9.0) 6-30-97 | Download Restore | Yes Yes | Note 5 Yes | No Yes |
| -01 7-23-97 | Download Restore | Yes Yes | Note 5 Yes | No Yes |
| -02 10-9-97 | Download Restore | Yes Yes | Note 5 Yes | No Yes |
| -03 11-25-97 | Download Restore | Yes Yes | Note 5 Yes | No Yes |
| -04 2-02-98 | Download Restore | Yes Yes | Note 5 Yes | No Yes |

| Software/EPROM Features for 9011 and 9012 (186) cpus Processor P/N: 502-04071-XX (STD 9011 AC), 502-04073-XX (Turbo 9012 AC), 502-04080-XX (STD 9011 DC), 502-04081-XX (Turbo 9012 DC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EPROM: 96742-XX | -00 | -01 | -02 | -03 | -04 |
| PiCPro 9.X Project Manager SERCOS Block I/O Flash load w/out stopping scan | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No |
| PPfW 11.0 and higher does not support the 186 processors |  |  |  |  |  |

* PiCPro 10.0 for Windows does not support PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

The MMC, 94X and 904X processors do not have EPROMs.  Instead, the system information is stored in flash memory resident on the processor.  The flash memory can be loaded to the processor via the PiCPro Utility called SENDHEX.  To load flash memory, use the following procedure:

- Turn off the PiC or MMC control.

- Connect the PiCPro cable to the PiCPro port and to the PC.

- For PiCPro Utilities: Type SENDHEX <filename> where <filename> is the name of the system file you wish to load.  For PiCPro for Windows version 11.0 and later: use the Online | Download Hex menu selection and the subsequent file selection dialog (when no .ldo is open); PPfW prompts for when to turn off and on the power to the PiC or MMC.

- Press Enter.

- Turn on the PiC control.

- The file will be loaded.

| Software/FLASH Compatibility Chart for 941, 943, 945 and 947 (486) CPUs Processor P/N: 502-04111-XX (Turbo2 941), 502-04011-XX (Turbo2 943), 502-03994-XX (Turbo3 945), 502-04112-XX (Turbo3 947) New Processor P/N: M.1016.9626 (Turbo2 941 w/o Block I/O), M.1016.9627 (Turbo2 941 with Block I/O), M.1016.9508 (Turbo2 943 w/o Block I/O), M.1016.9510 (Turbo2 943 with Block I/O), M.1016.9491 (Turbo3 945), M.1016.9629 (Turbo3 947) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 95858-XX P94Xxx.hex OR PiC900 94X CPU Rxxxx.hex |  |  | PiCPro 8.X |  | PiCPro 9.X |  | PPfW 10.X |  | PPfW 11.0 |  | PPfW 12.0 |  | PPfW 13.0 |  | PPfW 14.0 |  | PPfW 14.1 |  | PPW 15.0 |  | PPW 15.1 |
| -00 (8.0) 01-26-96 | Download Restore | Yes Yes |  | Yes Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -01 09-27-96 | Download Restore | Yes Yes |  | Yes Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -02 (9.0) 70-23-97 | Download Restore | Yes Yes |  | Yes Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -03 10-30-97 | Download Restore | Yes Yes |  | Yes Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -04 11-25-97 | Download Restore | Yes Yes |  | Yes Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -05 06-18-98 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -06 (10.1) 04-02-99 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -07 (10.2) 04-07-99 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  | Note 5 Yes |  |
| -08 (11.0) 12-16-99 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -09 (12.0) 01-16-01 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -10 (12.0)SP1 04-26-01 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -11 (13.0) 05-20-02 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -12 (13.0)SPEC5 04-25-02 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -13 (14.0) 10-07-03 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -14 (14.1) 05-12-04 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |
| -15(15.0) 07-05-05 | Download Restore | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Note 5 Note 5 |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  | Yes Yes |  |

| Software/FLASH features for 941, 943, 945 and 947 (486) CPUs Processor P/N: 502-04111-XX (Turbo2 941), 502-04011-XX (Turbo2 943), 502-03994-XX (Turbo3 945), 502-04112-XX (Turbo3 947) New Processor P/N: M.1016.9626 (Turbo2 941 w/o Block I/O), M.1016.9627 (Turbo2 941 with Block I/O), M.1016.9508 (Turbo2 943 w/o Block I/O), M.1016.9510 (Turbo2 943 with Block I/O), M.1016.9491 (Turbo3 945), M.1016.9629 (Turbo3 947) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 95858-XX P94Xxx.hex hex OR PiC900 94X CPU Rxxxx.hex | -00 | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 | -09 | -10 | -11 | -12 | -13 | -14 | -15 |
| PiCPro 8.0X On-Line Edit UDFB Animation, View and Force Enhancement | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| PiCPro 9.X Project Manager SERCOS Block I/O Flash load w/out stopping scan | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No | No | No | No | No | No | No | No | No |
| PPfW 11.0 New Project Manager | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| PPfW 12.0 MMC for PC Enlarged data space | No No | No No | No No | No No | No No | No No | No No | No No | No No | No No | No Yes | No Yes | No Yes | No Yes | No Yes | No Yes |
| PPfW 13.0 # Structured Text | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes | Yes |
| PPfW 13.0 SP3 # DLS # J-K Thermocouple | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes |
| PPfW 14.0/14.1 MMC Plus MMC Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |
| PPfW 15.1 MMC Digital Smart Drive | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

* PiCPro 10.0 for Windows does not support PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/FLASH Compatibility Chart for 9041 and 9043 (486) cpus Processor P/N: 502-04104-XX (STD 9041 AC), 502-04125-XX (Turbo 9043 AC), 502-04110-XX (STD 9041 DC), 502-04126-XX (Turbo 9043 DC) New Processor P/N: M.1016.9615 (STD 9041 w/o Block I/O), M.1016.9617 (STD 9041 with Block I/O), M.1016.9677 (Turbo 9043 w/o Block I/O), M.1016.9677 (Turbo 9043 with Block I/O) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 97493-XX P904Xxx.hex OR PiC90 904X CPU Rxxxx.hex |  | PiCPro 9.2 | PiCPro 9.3 | PPfW 10.0 | PPfW 11.0 | PPfW 12.0 | PPfW 13.0 | PPfW 14.0 | PPfW 14.1 | PPW 15.0 | PPW 15.1 |
| -00 (9.2) 05-26-98 | Download Restore | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes |
| -01 06-03-98 | Download Restore | Yes Yes | Yes Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes |
| -02 (10.1) 04-02-99 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes |
| -03 (10.2) 04-07-99 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes | Note 5 Yes |
| -04 (11.0) 12-16-99 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -04 (11.0)SP1 02-25-00 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -05 (12.0) 01-16-01 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -06 (NA) |  |  |  |  |  |  |  |  |  |  |  |
| -07 (12.0)SP1 04-26-01 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -08 (13.0) 05-20-02 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -09 (13.0)SPEC5 04-25-03 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -10 (14.0) 10-07-03 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -11 (14.1) 05-12-04 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -12 (15.0) 07-05-05 | Download Restore | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Note 5 Note 5 | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |

| Software/FLASH Features for 9041 and 9043 (486) cpus Processor P/N: 502-04104-XX (STD 9041 AC), 502-04125-XX (Turbo 9043 AC), 502-04110-XX (STD 9041 DC), 502-04126-XX (Turbo 9043 DC) New Processor P/N: M.1016.9615 (STD 9041 AC w/o Block I/O), M.1016.9617 (STD 9041 AC with Block I/O), M.1016.9677 (Turbo 9043 AC w/o Block I/O), M.1016.9677 (Turbo 9043 AC with Block I/O) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 97493-XX P904Xxx.hex Or PiC90 904X CPU Rxxxx.hex | -00 | -01 | -02 | -03 | -04 | -05 | -07 | -08 | -09 | -10 | -11 | -12 |
| PiCPro 9.2 Project Manager SERCOS Block I/O Flash load w/out stopping scan | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| PiCPro 9.3 Project Manager SERCOS Block I/O Flash load w/out stopping scan | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Yes Yes Yes Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |
| PPfW 10.X PPfD Project Manager* | No | No | No | No | No | No | No | No | No | No | No | No |
| PPfW 11.0 New Project Manager | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| PPfW 12.0 MMC for PC Enlarged data space | No No | No No | No No | No No | No No | No No | No No | No No | No No | No No | No No | No No |
| PPfW 13.0 Structured Text | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes | Yes |
| PPfW 13.0 SP3 DLS J-K Thermocouple | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes |
| PPfW 14.0/14.1 MMC Plus MMC Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |
| PPfW 15.1 MMC Digital Smart Drive enhancements | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

* PiCPro 10.0 for Windows does not support PPfD Project Manager; therefore compressed *.G&L files cannot be extracted.

| Software/FLASH Compatibility Chart for MMC and MMC Plus (486) cpus Processor P/N: 503-26606-XX ((-00 is SERCOS; -20 is 2 axis; -40 is 4 axis) New Processor P/N: M.1017.3772 (2 axis), M.1017.3774 (4 axis), M.1302.7095 (2 axis PLUS), M.1302.7096 (4 axis PLUS), M.1017.3770 (MMC-S8 8 axis SERCOS), M.1302.7479 (MMC-S8 PLUS 8 axis SERCOS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98161-XX MMCxx.hex Or MMC-Ax Analog MMC Rxxxx.hex Or MMC-Sx SERCOS MMC Rxxxx.hex |  |  | PPfW 10.2 |  |  | PPfW 11.0 |  |  | PPfW 12.0 |  |  | PPfW 13.0 |  |  | PPfW 14.0 |  |  | PPfW 14.1 |  |  | PPW 15.0 |  |  | PPW 15.1 |  |  |
| -00 (10.2) 05-21-99 |  | Download Restore |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |
| -01 (11.0) 12-16-99 | Download Restore |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |
| -02 (12.0) 01-16-01 | Download Restore |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |  | Note 5 Yes |  |
| -03 (12.0)SP1 04-26-01 | Download Restore |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |
| -04 (13.0) 05-20-02 | Download Restore |  |  | Note 5 Note 5 |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |
| -05 (13.0)SPEC5 04-25-03 | Download Restore |  |  | Note 5 Note 5 |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |
| -06 (14.0) 10-07-03 | Download Restore |  |  | Note 5 Note 5 |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |
| -07 (14.1) 05-12-04 | Download Restore |  |  | Note 5 Note 5 |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |
| -08 (15.0) 07-05-05 | Download Restore |  |  | Note 5 Note 5 |  |  | Note 5 Note 5 |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |  | Yes Yes |  |

| Software/FLASH Features for MMC (486) cpus Processor P/N: 503-26606-XX (-00 is SERCOS; -20 is 2 axis; -40 is 4 axis) New Processor P/N: M.1017.3772 (2 axis), M.1017.3774 (4 axis), M.1302.7095 (2 axis PLUS), M.1302.7096 (4 axis PLUS), M.1017.3770 (MMC-S8 8 axis SERCOS), M.1302.7479 (MMC-S8 PLUS 8 axis SERCOS) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98161-XX MMCxx.hex Or MMC-Sx SERCOS MMC Rxxxx.hex or MMC-Ax Analog MMC Rxxxx.hex | -00 | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 |  |
| PPfW 10.2 MMC | Yes | Yes | Yes | Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |  |
| PPfW 11.0 New Project Manager | Note 5 | Yes | Yes | Yes | Note 5 | Note 5 | Note 5 | Note 5 | Note 5 |  |
| PPfW 12.0 MMC for PC Enlarged data space | No No | No No | No No | No No | No No | No No | No No | No No | No No |  |
| PPfW 13.0 Structured Text | Note 5 | Note 5 | Note 5 | Note 5 | Yes | Yes | Yes | Yes | Yes |  |
| PPfW 13.0 SP3 DLS J-K Thermocouple | NA Note 5 | NA Note 5 | NA Note 5 | NA Note 5 | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes |  |
| PPfW 14.0/14.1 MMC Plus MMC Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | Yes NA | Yes NA | Yes NA |  |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |  |
| PPfW 15.1 MMC Digital Smart Drive | n/a | NA | NA | NA | NA | NA | NA | NA | NA |  |

| Software/FLASH Compatibility Chart for MMC for PC SERCOS (486) cpu Processor P/N: M.1300.5973 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98696-XX MMCPCSxx.hex Or MMC for PC-Sx SERCOS Rxxxx.hex |  | PPfW 12.0 | PPfW 13.0 | PPfW 14.0 | PPfW 14.1 | PPW 15.0 | PPW 15.1 |
| -00 (NA) |  |  |  |  |  |  |  |
| -01 (12.0) 01-16-01 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -02 (12.0)SP1 04-26-01 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -03 (12.0)SP2 10-26-01 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -04 (13.0) 05-20-02 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -05 (13.0)SP1 08-05-02 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -06 (13.0)SP3 10-22-02 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -07 (13.0)SP4 03-19-03 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -08 (13.0)SPEC5 04-25-03 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -09 (14.0) 10-07-03 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -10 (14.1) 05-12-04 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -11 (15.0) 07-05-05 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -12 (15.1) 9-12-05 | Download Restore | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |

| Software/FLASH Features for MMC for PC SERCOS (486) cpu Processor P/N: M.1300.5973 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98696-XX MMCPCSxx.hex Or MMC for PC-Sx SERCOS R0512.hex | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 | -09 | -10 | -11 | -12 |  |
| PPfW 12.0 MMC for PC Enlarged data space | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |  |
| PPfW 12.0 SP1 32 Axes SERCOS | No | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| PPfW 13.0 Structured Text | No | No | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| PPfW 13.0 SP3 DLS J-K Thermocouple | No Note 5 | No Note 5 | No Note 5 | No Yes | No Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |  |
| PPfW 14.0 MMC Plus MMC Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |  |
| PPfW 14.1 Upload All SERCOS slaves | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 | Yes | Yes | Yes |  |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |  |
| PPfW 15.1 MMC Digital Smart Drive enhancements | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |  |

| Software/FLASH Compatibility Chart for MMC for PC Analog (486) cpu Processor P/N: M.1300.5974 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98817-XX MMCPCAxx.hex Or MMC for PC Analog Rxxxx.hex |  | PPfW 12.0 |  | PPfW 13.0 | PPfW 14.0 | PPfW 14.1 | PPW 15.0 | PPW 15.1 |
| -00 (NA) |  |  |  |  |  |  |  |  |
| -01 (12.0) 01-16-01 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -02 (12.0)SP1 04-26-01 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -03 (13.0) 05-20-02 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -04 (13.0)SP1 08-05-02 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -05 (13.0)SP4 03-19-03 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -06 (13.0)SPEC5 04-25-03 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -07 (14.0) 10-07-03 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -08 (14.1) 05-12-04 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -09(15.0) 07-05-05 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |
| -10(15.1) 09-22--05 | Download Restore |  | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |

| Software/FLASH Features for MMC for PC Analog (486) cpu Processor P/N: M.1300.5974 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLASH FILE: 98817-XX MMCPCAxx.hex Or MMC for PC Analog Rxxxx.hex | -01 | -02 | -03 | -04 | -05 | -06 | -07 | -08 | -09 | -10 |  |
| PPfW 12.0 MMC for PC Enlarged data space | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes | Yes Yes |  |
| PPfW 13.0 Structured Text | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| PPfW 13.0 SP3 DLS J-K Thermocouple | NA Note 5 | NA Note 5 | NA Note 5 | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes | NA Yes |  |
| PPfW 14.0/14.1 MMC Plus MMC Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |  |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA | No NA |  |
| PPfW 15.1 MMC Digital Smart Drive enhancements | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |  |

| Software/FLASH Compatibility Chart for MMC digital dRIVE RESIDENT CONTROL |  |  |  |  |
| --- | --- | --- | --- | --- |
| MMC-SD-xxx-D Digital MMC Smart Drive Rxxxx |  | PPW 15.0 |  | PPW 15.1 |
| -00(15.0) 07-05-05 | Download Restore |  | Yes Yes | Yes Yes |
| -01(15.1) 09-08-05 | Download Restore |  | Yes Yes | Yes Yes |
| -02(15.1-SP1) 05-26-06 | Download Restore |  | Yes Yes | Yes Yes |
| -02(15.1-SP2) 07-07-06 | Download Restore |  | Yes Yes | Yes Yes |
| -02(15.1-SP3) 07-18-06 | Download Restore |  | Yes Yes | Yes Yes |

| Software/FLASH Features for MMC ditital DRIVE RESIDENT CONTROL |  |  |  |
| --- | --- | --- | --- |
| MMC-SD-xxx-D Digital MMC Smart Drive Rxxxx | -00 | -01 | -02 |
| PPfW 15.0 MMC Drive Resident Control MMC Digital Smart Drive | No NA | No NA | No NA |
| PPfW 15.1 MMC Digital Smart Drive enhancements | NA | NA | NA |
| PPfW 15.1-SP1/ PPfW 15.1-SP2/PPfW 15.1-SP3 Notch Filter 10,000 Line Encoder | NA NA | NA NA | NA NA |

MMC/PiC90/900 hardware COMPATIBILITY ISSUES

Several issues surround the subject of MMC/PiC CPU replacements as well as several other MMC and PiC90/900 modules. When a requested replacement part for an existing system appears in the left column and the exact part number is no longer available, the customer may be referred to technical support if compatibility issues are not clear.  No issues exist with new systems used with current firmware and software.

CPU modules

Three issues surround the matter of MMC/PiC CPUs: Hardware, Firmware, and Software.  The Hardware component provides a CPU ID, which identifies with a Hardware Declaration required in PiCPro.  Earlier versions of PiCPro do not have the CPU declaration to identify newer CPUs.  Therefore, as newer CPUs are released into production, later versions of PiCPro are required to support hardware identification.

Firmware is another issue.  Firmware is updated to support new software features.  Some cross-compatibility exists between older firmware and newer software, i.e., PicPro 9.3 will work with version 400 firmware; however, Block I/O is not supported.  To ensure all features are supported, you need the latest software and firmware in your system.   The first section of this document has the details regarding software/EEPROM compatibility.

MMC CPU modules

Below is a list of older versus newer MMC CPUs.  The right column consists of the latest MMC CPU releases.  CPUs listed in the left column should be referred to technical support for replacement parts on existing systems.  No issues exist with new systems purchased with current software.

If you have this CPU:						You can use this CPU:

M.1017.3774	MMC-S8 8 axis SERCOS			M.1302.7479 MMC-S8 Plus 8 axis SERCOS1

Note 1:  PiCPro 14.0-SP2 (or higher) is required for MMC-S8 Plus CPU’s.

Note2: PicPro 15.0 (or higher) is required for MMC-DIGITAL DRIVE RESIDENT CONTROLS

PIC90/900 CPU modules

Below is a list of older versus newer PiC90/900 CPUs.  The right column consists of the latest CPU releases.  CPUs listed in the left column should be referred to technical support for replacement parts on existing systems.  No issues exist with new systems purchased with current software.

If you have this CPU:						You can use this CPU:

502-03510-11	PiC900 8 MHz Standard 64K/0K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03510-21	PiC900 8 MHz Standard 64K/64K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03510-31	PiC900 8 MHz Standard 64K/256K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03510-41	PiC900 8 MHz Standard 256K/0K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03510-51	PiC900 8 MHz Standard 256K/64K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03510-61	PiC900 8 MHz Standard 256K/256K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03941-11	PiC900 10 MHz Standard 911 128K/0K3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03941-21	PiC900 10 MHz Standard 911 128K/128K3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03941-51	PiC900 10 MHz Standard 911 384K/256K3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-11	PiC900 16 MHz Turbo 64K/0K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-21	PiC900 16 MHz Turbo 64K/64K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-31	PiC900 16 MHz Turbo 64K/256K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-41	PiC900 16 MHz Turbo 256K/0K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-51	PiC900 16 MHz Turbo 256K/64K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03638-61	PiC900 16 MHz Turbo 256K/256K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03963-11	PiC900 20 MHz Turbo 912 128K/0K3 		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03963-21	PiC900 20 MHz Turbo 912 128K/128K3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03963-31	PiC900 20 MHz Turbo 912 128K/128K ARCNET3	502-04111-11 PiC900 16 MHz 941 512K/256K2

502-03963-41	PiC900 20 MHz Turbo 912 384K/128K ARCNET3	502-04111-11 PiC900 16 MHz 941 512K/256K2

502-03963-51	PiC900 20 MHz Turbo 912 384K/256K3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03963-61	PiC900 20 MHz Turbo 912 384K/256K ARCNET3	502-04111-11 PiC900 16 MHz 941 512K/256K2

502-03794-40	PiC900 Turbo2 SX 16 MHz 256K/0K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03794-60	PiC900 Turbo2 SX 16 MHz 256K/256K1,3		502-04111-01 PiC900 16 MHz 941 512K/256K2

502-03814-40	PiC900 Turbo2 DX 25 MHz 256K/0K1,3		502-04011-01 PiC900 32 MHz 943 512K/256K4,5

502-03814-60	PiC900 Turbo2 DX 25 MHz 256K/256K1,3		502-04011-01 PiC900 32 MHz 943 512K/256K4,5

502-03846-03	PiC90 8 MHz Standard 128K/64K3		502-04104-00 PiC90 16 MHz 9041 256K/128K2

502-03846-23	PiC90 8 MHz Standard 128K/64K ARCNET3	502-04104-10 PiC90 16 MHz 9041 256K/128K2

502-03853-03	PiC90 16 MHz Turbo 256K/128K3		502-04125-00 PiC90 32 MHz 9043 256K/128K2

502-03853-23	PiC90 16 MHz Turbo 256K/128K ARCNET3	502-04125-10 PiC90 32 MHz 9043 256K/128K2

502-04073-20	PiC90 20 MHz 9012 256K/128K ARCNET/BIO3	502-04104-10 PiC90 10 MHz 9041 256K/128K2

Note 1:  On old style CPU’s with daughter boards installed, use -11 version of new CPU.

Note 2:  PiCPro 9.2 (or higher) is required for 941, 947, 9041, and 9043 processors.

Note 3:  All older CPU types are stocked with the latest available EEPROM revision.  PiCPro 8.0 or later is recommended to ensure reliability with new EEPROMs.  EEPROM downgrades are available for users with PiCPro 7.1 or earlier.

Note 4:	943 CPUs require PiCPro 9.1 or higher when Block I/O is NOT used.

Note 5:	943 CPUs require PiCPro 9.2 or higher when Block I/O is used.

CSM Modules on PiC 900 Racks

Does the customer have a non-EMC compliant rack?  This can be determined by the part number of the CSM module (this module is mounted in slot one).  EMC compliant Modules have the EMC compliant stamp on the label.  Intermittent operation may occur when a non-EMC compliant CSM is replaced with an EMC compliant CSM.  Refer to Appendix “O” in the PiC900 Hardware Manual for details regarding use of EMC and NON-EMC Modules.

For NON-EMC Racks:						You can use this CSM:

502-03512-00, 01	CSM-50					502-04149-00

502-03813-00, 01	CSM-60					502-04151-00

502-03732-00, 01	RSM-50					502-04149-00

502-03817-00, 01	RSM-60					502-04151-00

For EMC Racks:

502-03512-02		CSM-50

502-03813-02		CSM-60

502-03732-02		RSM-50

502-03817-02		RSM-60

PiC REMOTE RACK COMMUNICATIONS

Reliability issues exist when using the following hardware/software combinations:

1.  PPfD 9.x used with 502-03638-41R2 PiC CPU modules

186 16Mhz CPU (502-03638-41 R2) does not run with Remote IO Communications Daughter Board when used with PiCPro 9.x .  502-03638-41 R4 must be used with PiCPro 9.x when Remote IO is used.

2.  New style PiC94x 486DX CPUs with older revisions of PiC IO driver modules

IO Driver module 502-03657-00 R5 and under causes scan loss when used with 94x CPUs.  IO Driver Module 502-03657-03 or later must be used with 94x CPUs.

PiC Communication Modules

There are firmware compatibility issues between PiC CPUs and PiC Communication Modules. A timing error exists in the Communication module revs -00, -01. The first byte received by a channel is interpreted as an error.  A firmware change in the –03R2 Communication module corrects the problem.

If you use the -00, -01, 02, 03R1 Communication Modules with version 400 CPU firmware, communications will work normally.

Conversely, if you use the -00, -01, 02, 03R1 Communication Modules with version 500 CPU firmware, an error occurs on the first Read attempt after power-up and first scan.  The error will clear when the scan is reset.  A firmware upgrade on the Communication Module is necessary for correct operation.

If you have this Comm. Module:					You can use this CPU firmware:

502-03676-00, -01, -02, -03R1  4 ch Comm Module			Version 400 or lower

502-03676-20, -21, -22, -23R1  2 ch Comm Module			Version 400 or lower

502-03676-03R2 and up	        4 ch Comm Module			Version 500 or Higher

502-03676-23R2 and up	        2 ch Comm Module			Version 500 or Higher

PiC Feedback Modules

The shield terminals on newer, EMC compliant Feedback Modules are terminated through the backplane.  This requires a wiring change in the module terminal strip.  Removal of the wire from the shield terminal to SPG is necessary in order to prevent ground loops.  This will prevent noise induced feedback errors.

Non-EMC Feedback Modules:					EMC compliant Feedback Modules:

502-03552-00,01	4 Ch Resolver	502-03552-02,06

502-03552-20,21	2 Ch Resolver	502-03552-22,26

502-03722-00,01	12 Ch Resolver	502-03722-02,06

502-03782-00,01	4 Ch Encoder-Low	502-03782-02,06

502-03782-20,21	2 Ch Encoder-Low	502-03782-22,26

502-03786-00,01	4 Ch Encoder-High	502-03786-02,06

502-03786-20,21	2 Ch Encoder-High	502-03786-22,26

502-03839-00,02	Servo 2AO 3E 4AI	502-03839-03,07

502-03840-00,02	Servo 4AO 3E	502-03840-03,07

502-03840-20,22	Servo 3AO 2E	502-03840-23,27

502-03840-40,42	Servo2AO 1E	502-03840-43,47

502-03642-00,02	Analog input	502-03642-03,08

PiC Analog Output Modules

The shield terminals on newer, EMC compliant Feedback Modules are terminated through the backplane.  This requires a wiring change in the module terminal strip.  Removal of the wire from the shield terminal to SPG is necessary in order to prevent ground loops.  This will prevent noise induced feedback errors.

Non-EMC Feedback Modules:					EMC compliant Feedback Modules:

502-03518-00,01	8 Ch D/A	502-03518-02,07

502-03518-20,21	4 Ch D/A	502-03518-22,27

502-03681-00,01	4-20 ma Out	502-03681-02,07

Revision History

Date: 3-Jan-96

Author:  Sue Dorscheid

Changes:  Original

Date:  3-Apr-96

Author:  Sue Dorscheid

Changes:  Revised charts to include PiCPro 8.0 and associated EPROMs.

Date:  20-Oct-97

Author:  Sue Dorscheid and Mike Tennessen

Changes:  Revised charts to include PiCPro 9.0 and associated EPROMs and new CPUs.

Date:  10-Dec-97

Author:  Sue Dorscheid

Changes:  Revised charts to include PiCPro 9.1 and associated EPROMs.

Date:  03-Mar-99

Author: Mike Tennessen

Changes:  Added Compatibility Issues section

Date:  16-Mar-99

Author: Mike Tennessen

Changes:  Added superscript call outs to Notes on CPU list.  Corrected serial communications module number of available channels (2,4 channels).  Changed Network drive from Earth to Vulcan.  Added path information.

Date:  03-May-99

Author: Mike Tennessen

Changes:  Changed 32 MHz clock speed to 10 MHz for 9041 CPU.

Date:  28-Sep-99

Author: Mike Tennessen

Changes:  Added PiCPro for Windows issues.

Date:  30-Nov-99

Author: Dave Michalets

Changes:  Added CPU module names (e.g., SAVE, Turbo); added recent firmware changes.

Date:  21-Feb-00

Author: Mike Tennessen

Changes:  Added 503-03794-XX to statement on Page 2:

PiCPro for Windows version 11.0 is only for CPUs with 486DX processors (as 11.0 introduced the 32-bit motion library) so it will not support the older CPUs (i.e., none of the 186 or 486SX processors).

Added “PiC REMOTE RACK COMMUNICATIONS” to Page 21

Date:  16-Mar-00

Author: Dave Michalets

Changes:  Added an extra page for better odd/even presentation when printed double-sided.  Added a few changes for PPfW 11.0 references.

Date:  25-Mar-02

Author:  Jerry Paveglio

Changes:  Brought things up to date through V12.0SP2.

Date:  10-Oct-03

Author:  Jerry Paveglio

Changes:  Brought things up to date through V14.0.

Date:  1-Dec-03

Author:  Kevin Kurek

Changes:  Added PPfW V14.0-SP1.

Date:  26-Jan-04

Author:  Kevin Kurek

Changes:  Added CPU MMC-S8 Plus 8 axis SERCOS and updated for PPfW V14.0-SP2.  Added section MMC CPU modules.

Date:  14-May-04

Author:  Kevin Kurek

Changes:  Changed company name from Giddings & Lewis to G&L Motion Control, LLC and updated for PPfWV14.1.

Date:  27-June-05

Author:  David Ross

Changes:  Changed company G & L Motion Control Inc and updated for PPWV15.0.

Date:  14-July-05

Author:  Kevin Kurek

Changes:  Updated firmware and PiCPro release dates for V15.0.

Date:  10-Oct-05

Author:  David Ross

Changes:  Updated for PPW 15.1 release.

Date:  13-Apr-06

Author:  Kevin Kurek

Changes:  Updated for PPW 15.1-SP1 release.

Date:  7-Jul-06

Author:  Kevin Kurek

Changes:  Updated for PPW 15.1-SP2 release.

Date:  18-Jul-06

Author:  Kevin Kurek

Changes:  Updated for PPW 15.1-SP3 release.