GIDDINGS & LEWISâ

PiC Application Note

Document Number:  AN000019

Topic:  Shunt Power Calculations for Incremental Motion Applications

Scope

This PiC Application Note applies to all Giddings & Lewis Centurionä drives (DSM100, DS100, DS200, and AS100) and motors (SSM, FSM, and HSM).

Purpose

A typical incremental motion profile is shown in figure 1.  During the shaded portions power is returned from the motor to the drive module.  The drive module handles the excessive regenerated power by dissipating it in a shunt power resistor.  The drive shunt circuit is capable of a peak power and a continuous average power.  This note describes how the peak and average shunt power can be estimated for the typical incremental motion profiles as in figure 1.  Refer to the specific drive manual to determine the applicable maximum peak and average shunt power ratings.  Refer to the spreadsheet at the end of this document for drive bus capacitance, shunt turn-on and shunt power.

Peak Shunt Power

PeakShuntPower(Watts)=SpeedRPM∗(Torquelb−in)HP63,025∗746WattsHP

Where:	Speed = Motor speed in RPM for point A as obtained from SYSTEX.

Torque = Torque in lb-in for point B as obtained from SYSTEX.

Refer to figure 1 for point A and B definitions.

Average Shunt Power

AverageShuntPower(Watts)=PeakShuntPowerWatts∗(T1)2∗T2

Where:	Peak Shunt Power is calculated above.

T1 = Time to decelerate.

T2 = Total cycle time as defined in Figure 1.

Note: Consult factory for assistance if T1 > 2 Seconds, or for more complex or non-repetitive motion profiles.

Figure 1 - Typical Incremental Motion Profile

VELOCITY                                         Point A

TORQUE (CURRENT)

Point B

POWER

Point C

T1

T2

DC Bus Capacitance

Note: If you are using the Systex, the peak and continuous generating power data has already been calculated and can be used in the equations below. Systex also provides speed and torque data.
All motor data specs (KT and winding resistance) can be found in the Product Guide.

I2R losses = continuous current2 x motor winding resistance

I = continuous current (Amps) = continuous torque (for point D in figure above) ¸ KT

R = motor winding resistance (W)

Continuous Shunt Power (Watts) = continuous generating power -

I2R losses- bus capacitance absorption

bus capacitance absorption = C/2 (Vf2 - Vi2)

C = bus capacitance of the drive

Vi = initial bus voltage (VDC)

240 VAC input Þ initial bus voltage = 240 x 1.414 = 325 VDC

115 VAC input Þ initial bus voltage = 115 x 1.414 = 160 VDC

Vf = voltage level that shunt turns on (VDC)

# Example

Using Systex, the system solution is a DSM130 drive powered with 115 VAC and a 
HSM460 motor.  The application deceleration torque is 110 lb-in, the maximum speed is 1800 rpm and the continuous torque is 50 lb-in.  The time to decelerate (t1) is 0.75 seconds and the cycle time (t2) is 2.8 seconds.  HSM460 KT = 4.4 lb-in/A and winding resistance = 0.69 W.

=745´110´180063,025=2340 Watts= 2340´0.752´2.8=313 Watts

peak generating power

continuous generating 	power

I2R losses 			=(50¸4.4)2´0.69=89 Watts

=2730´10-62´4252−115´22=210 Watts

bus capacitance 	absorption

Continuous Shunt Power		=313 Watts −89 Watts −210 Watts

=14 Watts

DSM130 continuous shunt power = 50 Watts

Application continuous shunt power = 14 Watts

Drive continuous shunt power > Application continuous shunt power

DSM130 peak shunt power = 4500 Watts

Application peak shunt power = 2340Watts

Drive peak shunt power > Application peak shunt power

=> Drive can dissipate application shunt power, therefore no external shunt is required.

Table of Drive Shunt and Capacitance Values.

Revision History

Date:  02/21/96

Author:  Jeff Johnson

Changes:  Original

Date:  01/16/97

Author:  Don Seichter

Changes:  Added calculations for bus capacitance

Date:  [For each revision, enter the date revised,]

Author:  [the name of the person revising the document,]

Changes:  [and indicate what changes were made.]