GIDDINGS & LEWIS

PiC Application Note

Document Number:  AN000029

Topic : Downloading .ASC files to 2100 Series TCS’s

ASCII  file load through WINDOWS

Windows 3.11

Select Windows Accessories

Double click on Terminal

Select Settings

Select Communications: 	    Baud Rate  9600	   Data Bits 7      Stop Bits  2     Parity  Even  				    Flow Control    XON/XOFF   Connector    COM1:

Select File

Select Save -  File Name ASC_XFER.TRM          (saves file for future use)

Select Transfer

Select Send text file  browse to fine file and select OK

The file will transfer and screens load.  The status bar at the bottom of the screen shows progress.

After ASC_XFER.TRM has been saved once:

Select File

Select Open  ASC_XFER.TRM

Select Transfer

Select Send Text File browse to find file and select OK

Windows 95

Click on Start - Programs - Accessories - Hyperterminal

Double click on the Hyperterm icon

Enter the name for the new icon    ASC_XFER    and  OK

Under the phone number window  select the Connect using list and select Direct to Com 1 and OK

Enter Port Settings of:  Bits per second   9600

Data bits             7

Parity                 Even

Stop Bits            2

Flow Control     XON/XOFF

Select OK

Select File - Properties - Settings - ASCII Setup

Check the box for: Echo typed characters locally

Select Transfer - Send Text File...

Select Files of type: All files (*.*)

Select Look in:  and browse to find your .asc file

Double click on the .asc file you wish to send or (highlight the .asc and select OPEN)

DOS  .ASC file copy

To copy an ASCII file to the TCS, enter two commands (from DOS)

Mode COM1: 9600,e,7,2,p

Copy Axxx.ASC COM1:

These file do not have a page number associated with them.  For example, the filenames for application 5 would be “A0005.BN1” and “A0005.ASC”.

The .ASC file is generated in PiCPage by doing a File - Generate Ascii file.

A new PAGEUDFB function block must be used for the latest version of 2100 Series touchscreens, or the correction can be made to the existing application PAGEUDFB function block by:

Enter the Software Declarations in the PAGEUDFB that is used.

Search for the declaration TS_STR0[10].

Change the software declaration string size of TS_STR0 from [10] to [100].

Do a UDFB build and save to the appropriate library.

New cabling  (cabling changes effect PiC 900 cables only)

PiC 9/90/900 Cable

2180/2185			DB-9S

3 RD				   3 TD

5 TD				   2 RD

1 GND				   5 GND

4 RTS				   6 RTS

8  CTS

Hood				Hood

Pigtail (to PiCPage)  Cable

2180/2185			DB-25S

3 RD				   3 TD

5 TD				   2 RD

1 GND				   5 GND

4 RTS				   11 RTS

4  CTS

20 CTS

Hood				Hood

Revision History

Date:  10 DEC 96

Author:  Rick Ruotsala

Changes:  Original

Date:  [For each revision, enter the date revised,]

Author:  [the name of the person revising the document,]

Changes:  [and indicate what changes were made.]