GIDDINGS & LEWIS

PiC Application Note

Document Number:  AN000035

Topic: Converting from a DS to a DSM Drive.

# Scope

This Application Note applies to the following Giddings & Lewis™ servo amplifiers:

- Centurion™ Line DSM100 and MicroDSM servo amplifiers from Giddings & Lewis

- Centurion™ Line DS100

# Purpose

This Application Note provides information for applications converting from the DS series to the DSM series of servo amplifiers.

Body

The DSM servo amplifier can be used in all situations where a DS servo amplifier was used in the past.  DSM servo amplifiers were designed with upgrade in mind.  The servo amplifier and motor performance are the same as the DS servo amplifier if the same Gain parameters are used.  The DSM servo amplifier supports all the different types of motors previously run by the DS servo amplifier.

Mechanically the foot print of the DS and DSM100 series servo amplifiers are the same.  The DSM servo amplifier will bolt into the same place as the DS servo amplifier.

There are some things that are different about the DSM servo amplifier.  The Electrical  input connections on the DSM servo amplifier are 24 V source inputs.  The DS servo amplifier used sinking inputs.  Therefore, a relay may be required on the inputs listed below.  See the Wiring Conversion Schematic for pin to pin connections.

Enable - Drive enable

RST -     Drive reset

FAC -    Forward current enable

RAC -    Reverse current enable

Note:  Normally the RAC and FAC inputs were always enabled to allow forward and reverse motor directions, therefore these inputs are no connects when converting to the DSM drive.  If  motor direction is to be inhibited then connect RAC or FAC to a selectable drive input in the same manner as connecting to the Enable and Reset inputs.  The selectable drive input must also then be set up correctly in the DSM drive via the DSMPro software.

The cabling connecting these inputs is also different.  The connectors on the DS servo amplifier were a Molex style of connector.  The DSM servo amplifier uses a mini-D style

of connector. Giddings and Lewis supplies a terminal strip converter for these connections.  There are two connectors required:

J1 terminal strip converter P/N: M.1016.9545  (old P/N: 401-04051-00)

J2 terminal strip converter P/N: M.1016.9581  (old P/N: 401-04076-00)

The communication protocol also changed from the DS to the DSM software. DSMPro is a new software package and is required for servo amplifier setup.  DSMPro is a windows program.  A new cable is also required.  The part numbers are:

DSMPro Windows Software: M.1017.0586  (old P/N: 502-60036-00)

DSMPro Cable 10 ft.	          : M.1016.9514  (old P/N: 502-04020-10)

Changes to the communication protocol may also have an effect if using a PiC900 to communicate to a DS servo amplifier. In this event a new Application Specific Function Block has been created, M_DSMCOM, for communicating to the DSM servo amplifiers from a PiC900.  It will be necessary to reprogram the software using the M_DSMCOM function.  To obtain this new function order:

Motion ASFB Software P/N:  503-18955-00.

Note: the Motion ASFB’s are now included on the Applications CD with PiCPro v11.0 and later.

Note:  If multi-drop communications are being used to communicate to multiple DS servo amplifiers it will be necessary to connect a separate serial port to the DSM servo amplifier unless all DS servo amplifiers are replaced with DSM servo amplifiers.

Please refer to the DS, MicroDSM or DSM servo amplifier installation manuals for additional information.  The DSM manuals can be viewed and downloaded from the Giddings and Lewis web site at:

http://www.glcontrols.com/downloads/technicalinformation/technical_info.asp

# Centurion DS to DSM Drive Wiring Conversion

DS Drive				DSM Drive

J2 Motor Encoder

J2-20,  TS (-)

J2-19,  TS (+)

J2-2,  ECOM

J2-1,  EPWR

J2-GD,  SPG

J2-16,  ABS

J2-12,  I (-)

J2-11,  I (+)

J2-10,  B (-)

J2-9,  B (+)

J2-8,  A (-)

J2-7,  A (+)

J2-4,  SENSE (-)

J2-3,  SENSE (+)

# J1 Controller

J1-GD,  SPG

J1-12,  I (-)

J1-11,  I (+)

J1-10,  B (-)

J1-9,  B (+)

J1-8,  A (-)

J1-7,  A (+)

## Motor

R

S

T

GRND

J1-21,  RESET

L1

L2/N

GND

L1 AUX

L2/N AUX

J1-GD,  SPG

J1-25,  READY (-)

J1-24,  READY (+)

J1-6,  ISOLATED 24VDC COM

J1-22,  COMMAND (+)

J1-23,  COMMAND (-)

J1-20,  ENABLE

P4 Connector – Encoder In

P4-14,  TS (-)

P4-13,  TS (+)

P4-12,  COMMON

P4-11,  +5 VDC

P4-10,  SHIELD

P4-9,  ABS

P4-8,  I (-)

P4-7,  I (+)

P4-6,  B (-)

P4-5,  B (+)

P4-4,  A (-)

P4-3,  A (+)

P4-2,  COMMON

P4-1,  +5 VDC

# P3 Connector – Encoder Out

P3-7,  SHIELD

P3-6,  I (-)

P3-5,  I (+)

P3-4,  B (-)

P3-3,  B (+)

P3-2,  A (-)

P3-1,  A (+)

# P2 – I/O									       DS Drive

P2-10,  +24 VDC                  NC								R

P2-9,  OUT (-)           	          NC								S

P2-8,  OUT (+)	          NC								T

P2-7,  +24 VDC COM          NC								GND

See pages 4, 5 for interface circuits

P2-6,  RST (-)

P2-5,  RST (+)									AC

P2-4,  RAC (-)	          NC								AC

P2-3,  RAC (+)	          NC								GND

P2-2,  FAC (-)	          NC								AC AUX

P2-1,  FAC (+)	          NC								AC AUX

# P1 Connector - Controller

P1-9,  SHIELD

P1-8,  +24 VDC	          NC

P1-7,  SSO (-)

P1-6,  SSO (+)

P1-5,  +24 VDC COM

P1-4,  VCS (+)

P1-3,  VCS (-)

See pages 4, 5 for interface circuits

P1-2,  ENABLE (-)

P1-1,  ENABLE (+)

### DSM Drive Input Circuit Examples

Control

PiC OUTPUT 	     +24V DC	              J1	      DSM DRIVE INPUT

24V DC SINK

MODULE			                5      Isolated

+24v DC							    			               26

DCSINK1

CR1	         20, 21

DCCOM1

Control   		                       Isolated

24V COM                                       24V COM

Enable or Reset Drive Input Connected to a Sinking Output Fig. 1

PiC OUTPUT

24V DC SINK		       	              J1	      DSM DRIVE INPUT

MODULE

5      Isolated

Control      DCOUT1			       +24v DC						        +24V DC    			               26

DCSINK1

CR1	         20, 21

DCCOM1

Control   		                       Isolated

24V COM                                       24V COM

Enable or Reset Drive Input Connected to a Sinking Output Fig. 2

PiC OUTPUT

24V DC SOURCE	        Control 	              J1	      DSM DRIVE INPUT

MODULE		      +24V DC

DCL1

DCOUT1

20, 21

DCCOM1

6, 13

Control 	                       Isolated

24V COM                          24V COM

Enable or Reset Drive Input Connected to a Sourcing Output

DSM Drive Input Circuit Examples

J1	 DSM DRIVE INPUT

5      Isolated

+24v DC						   		             PB	         26

21

Isolated

24V COM

Reset Drive Input Connected to a Pushbutton Switch Fig. 1

J1	    DSM DRIVE INPUT

#### Control

+24V DC

PB

21

6, 13

Control	     Isolated

24V COM	    24V COM

Reset Drive Input Connected to a Pushbutton Switch Fig. 2

Consult the DSM or MicroDSM  servo amplifier installation manuals for additional drive input interface circuits.

Revision History

Date:  26-JAN-98

Author:  Don Seichter

Changes:  Original

Date:  05-Feb-01

Author:  Randy Bauer

Changes:  Added wiring conversion schematic.

Date:  [For each revision, enter the date revised,]

Author:  [the name of the person revising the document,]

Changes:  [and indicate what changes were made.]