GIDDINGS & LEWIS

PiC Application Note

Document Number:  AN000045

Topic: Converting from a DS to a DSA Drive.

IMPORTANT INFORMATION

When replacing a DS or a DSM drive with a DSA drive, it may be necessary to also replace the motor and / or the encoder cable.

DS drives and in some cases DSM drives utilized a method of start up commutation that is not supported by the DSA.  A signal called ABS from the motor was used for start up commutation.  It required only 1 wire in the encoder cable from motor to drive.

The DSA requires 3 - Hall Effect sensor signals from the motor to the drive for start up commutation.  There are 2 things to look out for because of this.

- SSM motors prior to June 1990 had no Hall Effect sensor outputs.

- The encoder cables did not have wires to the appropriate pins and there were no spares available for use.

Therefore you must check that the motor provides the appropriate signals and that the cables used are wired for them also.

SSM motors with a date code earlier than June 1990 did not have the Hall Effect sensors fitted.

In mid 1990 the Hall Effect sensors were fitted.  Check the motor date code on the name plate.

Call G&L Motion Control Technical Support if the date code is earlier than June 1990.

WARNING

Starting up a DSA drive without the motor Hall Effect sensors connected can result in a motor runaway condition.

# Scope

This Application Note applies to the following Giddings & Lewis™ servo amplifiers:

- Centurion™ Line DSA100 and MicroDSA servo amplifiers from Giddings & Lewis

- Centurion™ Line DS100

# Purpose

This Application Note provides information for applications converting from the DS series to the DSA series of servo amplifiers.

Body

The DSA servo amplifier can be used in all situations where a DS servo amplifier was used in the past.  DSA servo amplifiers were designed with upgrade in mind.  The servo amplifier and motor performance are the same as the DS servo amplifier if the same Gain parameters are used.  The DSA servo amplifier supports all the different types of motors previously run by the DS servo amplifier except SSM motors built prior to June 1990.

Mechanically the foot print of the DS and DSA100 series servo amplifiers are the same.  The DSA servo amplifier will bolt into the same place as the DS servo amplifier.

There are some things that are different about the DSA servo amplifier.  The Electrical input connections on the DSA servo amplifier are 24 V source inputs.  The DS servo amplifier used sinking inputs.  Therefore, a relay may be required on the inputs listed below.  See the Wiring Conversion Schematic for pin to pin connections.

Enable - Drive enable

RST -     Drive reset

FAC -    Forward current enable

RAC -    Reverse current enable

Note:  Normally the RAC and FAC inputs were always enabled to allow forward and reverse motor directions, therefore these inputs are no connects when converting to the DSA drive.  If motor direction is to be inhibited then connect RAC or FAC to a selectable drive input in the same manner as connecting to the Enable and Reset inputs.  The selectable drive input must also then be set up correctly in the DSA drive via the DSAPro software.

The cabling connecting these inputs is also different.  The connectors on the DS servo amplifier were a Molex style of connector.  The DSA servo amplifier uses a HD-Sub style of connector. . A new motor feedback cable can be ordered.  (Option 1: this is recommended since the wires may have become brittle with age). Or a terminal strip converter (Option 2) for these connections:

There are two connectors required Option 2:

J1 terminal strip converter P/N: M.1301.4061

J2 terminal strip converter P/N: M.1301.4039

The communication protocol also changed from the DS to the DSA software. DSAPro is a new software package and is required for servo amplifier setup.  A new cable is also required.  The part numbers are:

DSAPro Windows Software (a free download from glcontrols.com)

DSAPro Windows Software: M.1300.7655

DSAPro Cable 10 ft.	          : M.1016.9514

Changes to the communication protocol may also have an effect if using a PiC900 to communicate to a DS servo amplifier. In this event a new Application Specific Function Block has been created, M_DSACOM, for communicating to the DSA servo amplifiers from a PiC900.  It will be necessary to reprogram the software using the M_DSACOM function.  To obtain this new function order:

Motion ASFB Software P/N:  503-18955-00.

Note: the Motion ASFB’s are now included on the Applications CD with PiCPro v11.0 and later.

Note:  If multi-drop communications are being used to communicate to multiple DS servo amplifiers it will be necessary to connect a separate serial port to the DSA servo amplifier unless all DS servo amplifiers are replaced with DSA servo amplifiers.

Please refer to the DS and DSA servo amplifier installation manuals for additional information.  The DSA manuals can be viewed and downloaded from the Giddings and Lewis web site at:

http://www.glcontrols.com/downloads/technicalinformation/technical_info.asp

# Centurion DS to DSA Drive Wiring Conversion

DS Drive				DSA Drive

J2 Motor Encoder

J2-6,  Common

J2-11,  TS (+)

J2-6,  ECOM

J2-14,  EPWR

Cable Shell,  Shield

Not Available

J2-10,  I (-)

J2-5,  I (+)

J2-4,  B (-)

J2-3,  B (+)

J2-2,  A (-)

J2-1,  A (+)

Not Available

Not Available

# J1 Controller

Cable Shell,  Shield

J1-21,  I (-)

J1-20,  I (+)

J1-19,  B (-)

J1-18  B (+)

J1-17,  A (-)

J1-16,  A (+)

## Motor

R

S

T

GRND

J1-35,  RESET

L1

L2/N

GND

L1 AUX

L2/N AUX

Cable Shell,  Shield

J1-44,  READY (-)

J1-43,  READY (+)

J1-27,  ISOLATED 24VDC COM

J1-25,  COMMAND (+)

J1-26,  COMMAND (-)

J1-31,  ENABLE

P4 Connector – Encoder In

P4-14,  TS (-)

P4-13,  TS (+)

P4-12,  COMMON

P4-11,  +5 VDC

P4-10,  SHIELD

P4-9,  ABS

P4-8,  I (-)

P4-7,  I (+)

P4-6,  B (-)

P4-5,  B (+)

P4-4,  A (-)

P4-3,  A (+)

P4-2,  COMMON

P4-1,  +5 VDC

# P3 Connector – Encoder Out

P3-7,  SHIELD

P3-6,  I (-)

P3-5,  I (+)

P3-4,  B (-)

P3-3,  B (+)

P3-2,  A (-)

P3-1,  A (+)

# P2 – I/O									       DS Drive

P2-10,  +24 VDC                  NC								R

P2-9,  OUT (-)           	          NC								S

P2-8,  OUT (+)	          NC								T

P2-7,  +24 VDC COM          NC								GND

See pages 4, 5 for interface circuits

P2-6,  RST (-)

P2-5,  RST (+)									AC

P2-4,  RAC (-)	          NC								AC

P2-3,  RAC (+)	          NC								GND

P2-2,  FAC (-)	          NC								AC AUX

P2-1,  FAC (+)	          NC								AC AUX

# P1 Connector - Controller

P1-9,  SHIELD

P1-8,  +24 VDC	          NC

P1-7,  SSO (-)

P1-6,  SSO (+)

P1-5,  +24 VDC COM

P1-4,  VCS (+)

P1-3,  VCS (-)

See pages 4, 5 for interface circuits

P1-2,  ENABLE (-)

P1-1,  ENABLE (+)

### DSA Drive Input Circuit Examples

Control

PiC OUTPUT 	     +24V DC	              J1	      DSA DRIVE INPUT

24V DC SINK

MODULE			                29      Isolated

+24v DC							    			               30

DCSINK1

CR1	         31,35

DCCOM1

Control   		                       Isolated

24V COM                                       24V COM

Enable or Reset Drive Input Connected to a Sinking Output Fig. 1

PiC OUTPUT

24V DC SINK		       	              J1	      DSA DRIVE INPUT

MODULE

29      Isolated

Control      DCOUT1			       +24v DC						        +24V DC    			               30

DCSINK1

CR1	         31,35

DCCOM1

Control   		                       Isolated

24V COM                                       24V COM

Enable or Reset Drive Input Connected to a Sinking Output Fig. 2

PiC OUTPUT

24V DC SOURCE	        Control 	              J1	      DSA DRIVE INPUT

MODULE		      +24V DC

DCL1

DCOUT1

31,35

DCCOM1

27,28

Control 	                       Isolated

24V COM                          24V COM

Enable or Reset Drive Input Connected to a Sourcing Output

DSA Drive Input Circuit Examples

J1	 DSA DRIVE INPUT

29      Isolated

+24v DC						   		             PB	         30

35

Isolated

24V COM

Reset Drive Input Connected to a Pushbutton Switch Fig. 1

J1	    DSA DRIVE INPUT

#### Control

+24V DC

PB

35

27,28

Control	     Isolated

24V COM	    24V COM

Reset Drive Input Connected to a Pushbutton Switch Fig. 2

Consult the DSA or MicroDSA servo amplifier installation manuals for additional drive input interface circuits.

Revision History

Date:  26-JAN-98

Author:  Don Seichter

Changes:  Original

Date:  05-Feb-01

Author:  Randy Bauer

Changes:  Added wiring conversion schematic.

Date:  18-Sept-03

Author:  Don Seichter

Changes:  Updated to apply to DSA Drive.

Date:  22-Sept-05

Author:  John Boyle

Changes:  Added Hall Effect Sensor WARNING and improved explanation

Date:  [For each revision, enter the date revised,]

Author:  [the name of the person revising the document,]

Changes:  [and indicate what changes were made.]